Acids in the chamber, history by bob?

Research Report

HZ University of Applied Science

Thesis

Author:			Vincent Seijger

Company:		Maersk Line A/S

Esplanaden 50

1098 Copenhagen K

Denmark

Study:			Maritime Officer

Course:			Bachelor of Maritime Operations, TRB Thesis

College:		De Ruyter Academy

HZ University of Applied Sciences

Edisonweg 4

4382NW Vlissingen

Company supervisor:	Mr.  J.J. Rengenhart, Mr. R.M. de Bruijn

University supervisor:	Mr. J.P.C. Wubben, Mr. A. de Groot

Version:		2. Final

Date:			6th of July 2017

Place:			Vlissingen

Summary

The title of this report can’t be interpret otherwise, the Blending-on-Board unit, or BOB-unit, placed two years ago on board of Maersk Line vessel is a piece of technology that shows a high range of flexibility when it comes to sailing with different kinds of Sulphur levels in the used fuels on board.

After two years of running time, there existed a desire to evaluate whether the result were in such a positive way that placement of the unit on board of new built ships and existing ships is recommended.

The research started with the analyses of the data on fuel, lubrication and cylinder lubrication oil in order to draw a conclusion on the savings that are made by the installation of this unit. A major difference is made between short time savings and future savings.

In combination with this a critical analysis of the engines cylinder components will be made to give explanation and visualize current status of the engine. Along with other qualitative research methods conclusions can be drawn on the lifetime contribution of these components brought by the BOB-unit.

Before the field work was done a literature study gives the reader all background information that is needed for a good understanding of the BOB-unit and its’ (dis)advantages. Other information about common wear mechanisms happening in the chamber will be given in this chapter. Also attention was given to cold corrosion, a mechanism that cannot be avoided just by only proper lubrication.

Results in this report show that for short-time savings no conclusions can be taken, as the cylinder oil lubrication consumption has become a steady factor, while before placement a wide spread of consumption can be seen for this consumption. As the vessel was sailing on different routes and so on different schedules no conclusion were being drawn on the fuel savings that could be made. Such testing procedures must only be made when having steady running conditions on a certain engine load. Models showing that the consumption in the higher loads, above 20 MW, a decrease of the consumption was seen, but this is based on minor measurements. Minor measurements caused due to the low loads that is required by the present sailing schedule.

For long-time savings the major factor was remaining an optimum engine condition, which is shown to be accomplished after placement of the BOB-unit. In order to sail another 5 years or more with these ’98 built vessels it is imperative to keep the engine in the highest quality and therefore can be stated that BOB optimizes this. In this case high cost of critical equipment can be avoided, such as cylinder liners, pistons, piston rings etc.

These conclusions state that it is recommended placing BOB-units on board of Maersk Line ships, not only on the new built vessels, but also vessels which have a few years to remain in operation.

--------------------- This page intentionally left blank ---------------------

Preface

This thesis is performed by Vincent Seijger, student Maritime Officer at the “De Ruyter Academy” part of the HZ University of Applied sciences. The period from February to June 2017 he has followed an internship at Maersk Line A/S as a part of his 10 months of total sailing days. In this period he gained experience in order to obtain his Bachelor of Maritime Operations degree, and this thesis is required in order to obtain the degree.

This work is brought forward as a thesis in cooperation with HZ University of Applied Sciences and Maersk Line, but also brings forward the importance of giving contribution to research and asses the current technology that will give cost optimization and upgrading the lifetime of older ships.

This, in my opinion, is outstanding way of maintaining the sustainability of the present vessels, instead of breakdown maintenance, where the vessel will last until its’ end without proper maintenance. The form of condition based maintenance in combination with new technology is therefore a sustained effort.

In the 5 months on board of the Maersk Kimi I have gained a lot of knowledge about the BOB-unit and interpret of the cylinder condition data. I want to thank all the officers and crew on board for their support and contribution, with special attention for chief engineers J.F. Kok (CEO) and J. Harinck, and mentors on board J.J. Rengenhart (2/E) and R.M. de Bruijn (3/E). Also I want to thank F. Wubben and A. de Groot for their cooperation and guidance in the progress of this report.

PERMISSION TO USE

It is understood that any copying or publication or use of this thesis shall not be allowed without the written permission of the researcher. It is also understood that due recognition shall be given to the researcher and to the University in any scholarly use which may be made of any material in this research report.

Request for permission to copy or to make other use of material in this research in whole or part by anyone else than Maersk Line personnel, should be addressed to the following persons:

Student:

Vincent Seijger

E-mail: seij0001@hz.nl

Tel. Nr. +31615907730

School supervisor:

J.P.C. Wubben

E-mail: wubb0002@hz.nl

Tel. nr. +31 118 489 486

List of abbreviations

BOB: Blending on Board

MFT: Maersk Fluid Technology

CLO: Cylinder Lubrication Oil

ECA: Emission control Area

BN: Base Number

SSW: Sudden Severe Wear

ME: Main Engine

ACC: Adaptable Cylinder Oil Control

MCR: Maximum Continuous Rating

HSFO & LSFO: High resp. Low Sulphur Heavy Fuel Oil

HFO: Heavy Fuel Oil

OEM: Original Equipment Manufacturer

Index

1.	Introduction	1

1.1	Problem cause	1

1.2	Problem statement	1

1.3	Main- and sub questions	2

1.4	Research Objective	2

2.	Theoretical framework	3

2.1	Blending-On-Board unit principles	3

2.1.1	Basic principles of the CLO unit	4

2.2	Cylinder Oil	5

2.3	Wear mechanisms	7

2.3.1 Cold corrosion	7

2.3.2 Adhesive wear	8

2.3.3 Abrasive wear	8

3.	Research design	9

3.1 Research methods	9

3.1.1 Method for sub question 1	9

3.1.2 Method for sub question 2	9

3.2 Data collection	10

3.3 Reliability and Validity	10

4.	Results	12

4.1	Cylinder oil	12

4.1.1	Reduction of CLO consumption	12

4.1.2	Changes in Viscosity	15

4.1.3	Changes in engine average load	16

4.2	Cylinder performance results	17

4.2.1	Piston ring data	18

4.2.2	Cylinder performance	20

4.2.2.1	Inspections	20

4.2.2.2	Analysis	20

4.3	Cylinder liner temperatures	22

5.	Discussion	23

5.1	CLO measurement	23

5.2	Viscosity	23

5.3	Load changes	23

5.4	Performance	24

5.5	Cylinder liner temperature	24

5.6	Breakdown	24

6.	Conclusion and recommendations	25

6.1	Conclusion	25

6.2	Recommendations	26

Literature	27

List of figures	28

Appendices	28

Appendix 1: Cylinder lubrication matrix example	29

Appendix 2: Lubrication analyses report	31

Appendix 3: Tier I upgrade	32

Appendix 4: Piston ring  measurements	34

Appendix 5: Cylinder condition report	35

Figure 1.1: Blending on Board mixing unit.

Introduction

New technology on cylinder lubrication oil has resulted in a change on container vessels by the placement of a complete new system called Seamate® Blending-On-Board (BOB), developed by Maersk Fluid Technology (MFT). This unit mixes system lubrication oil with additives which creates lubrication oil (CLO) with the specifications of cylinder lubrication oil. This mix is specially formulated for the used fuel specifics, as they vary depending on the taken bunkers. After two years of operational time, the unit must be evaluated in order to see whether there is a return of investment.

## Problem cause

When the unit was installed MFT stated that fuel consumption will be reduced up to 1,5 %. This is actually possible by the continuous refreshment of the system oil, which results in less friction inside the engine. Another statement was that, because of the optimal composition of the CLO, 40% of reduction of lubrication oil will occur.

Cost reduction was also possible by this unit because it will reduce maintenance costs. The CLO has created an optimum composition that will reduce wear of components. Stated was that piston ring time-between-overhaul will be doubled as an example. Due to the lowest feed rate it will preserve the integrity of the piston ring’s surface.

Another consequent problem cause would be the compliance with the International Convention for the Prevention of Pollution from Ships, where the maximum emissions of Sulphur oxide, nitrogen oxide, particles matter and carbon dioxide are being set. The CLO is burnt inside the cylinder but does it have a different composition than the conventional CLO complying with the allowed in the Emission Control Area’s (ECA). Due to a lack of information on this subject and because the vessel does not sail in ECA’s, there will be no research on this item, but there will be discussed on what to measure and how to measure.

## Problem statement

Prof. ir. A.C. de Leeuw defines the definition of a problem as follows, “a subjective feeling of displeasure which the user has, combined with the desire to do something about it” (Leeuw, 1980). As Maersk Line installed the unit some years ago on the vessels, until now Maersk is not aware of the results end therefore exists the desire to evaluate it. With this evaluation, the company is capable to analyze whether this the makers claims are achieved, so recommendations can be made concerning the installation of the BOB unit on new to be built vessels with two-stroke engines and older ships which do not have the unit installed.

  - Main- and sub questions

For this research the following main question is drafted:

Has a Seamates’ Blending-On-Board unit (BOB) on board Maersk container vessels lead to a positive result by the supposed savings and maintenance optimization, and if not can optimization be achieved yet?

In order to give answers on this question the following sub questions will support this answer:

- How did placement of the BOB unit lead to a cost reduction on fuel and lubrication oil?

- In what way did the unit contribute in a positive influence on lifetime of engine components?

The diagram in figure 1.2 shows the relation between the questions asked and the subjects that will be researched in order to answer the main problem:

Has Seamates’ Blending-On-Board unit on board Maersk container vessels lead to a positive interest by the supposed savings and maintenance optimization?

Has Seamates’ Blending-On-Board unit on board Maersk container vessels lead to a positive interest by the supposed savings and maintenance optimization?

1. How did placement of the BOB unit lead to a cost reduction on fuel and lubrication oil?

1. How did placement of the BOB unit lead to a cost reduction on fuel and lubrication oil?

2. In what way did the unit contribute in a positive influence on lifetime of engine components?

2. In what way did the unit contribute in a positive influence on lifetime of engine components?

1.1 Fuel reduction

1.1 Fuel reduction

1.2 Lubrication oil

1.2 Lubrication oil

1.3 Cylinder oil

1.3 Cylinder oil

2.1 Piston ring analysis

2.1 Piston ring analysis

2.2 Engine wear

Figure 2.1 Cylinder schematic cross-section. Reprinted from Marinestudy.net, by author M.H. Dewan, 2015, Retrieved from http://marinestudy.net/cylinder-lubrication-system/. Copyright 2017 by Marine Study. Reprinted with permission.2.2 Engine wear

Figure 1.2 Internal relation between research questions

## Research Objective

This research will bring out an advice to Maersk Line Management about the success of the BOB-unit placement on board the Maersk Kimi, by giving an evaluation of the savings made and the elongation of the time-between-failures of the engine cylinder components.

Herewith Maersk Line Management can use the results of this case-study in order to review whether further operation and future placement is advised on the remaining ships and ships that will be new-built.

# Theoretical framework

In this chapter the theoretical background information of the research will be given. The first part will give an introduction and explanation about the installed bob-unit. In the second part will be discussed about cylinder oil in order to give answer to the first sub question, where furthermore the combustion is discussed and the prices of the oil. In paragraph 3 the relation between the BOB-unit and life time of components and there failures will be discussed, in order to come to the answer of the second sub question.

## Blending-On-Board unit principles

Figure 2.1 Cylinder schematic cross-section. Reprinted from Marinestudy.net, by author M.H. Dewan, 2015, Retrieved from http://marinestudy.net/cylinder-lubrication-system/. Copyright 2017 by Marine Study. Reprinted with permission.

Before the principle of the BOB-unit is discussed the principle of cylinder lubrication will be explained. Large two-stroke crosshead engines, such as the Sulzer 12RTA96C installed on the Maersk Kimi, do normally have separate cylinder lubrication (Kuiken, 2012). As later explained in 2.2, oil will be introduced into the cylinder and the oil must spread over the liner surface, in such a way that a complete film of oil is formed. Oil will enter the cylinder through injection holes, or quills, as can be seen in Figure 2.1.

Sulzer fitted an accumulator to the quill that provides a reserve of oil, which ensures oil is always delivered to the cylinder at the desired time. The lubricator which takes the oil from the CLO storage tank is a metered pump which supplies oil to the individual cylinder quills. The engine builder, Wärtsilä, aimed to deliver oil to the cylinder from each quill at each piston stroke and rely upon the pressure differential between the cylinder and the oil in the quill unit. (Griffiths, 2006)

Whereas cylinder oil was bunkered in the CLO storage tank in harbor, nowadays it is made from system lubrication oil (SLO) mixed with additives, this will be discussed yet  in 2.2 “Cylinder Oil”. There are different systems available at the market, but for the Wärtsilä Sulzer engines Maersk Fluid Technology designed the Seamate Blending-on-Board unit.

Figure 2.2: Visualization of cylinder oil system

The system, as visualized in Figure 2.2, is basically the same as the conventional unit, except for the fact that the blender is connected with the used system oil and the previously CLO tank is now filled with high BN CLO product. Before two kinds of CLO were present on board for High Sulphur HFO (HSFO) and Low Sulphur (LSFO), where now the other tank is filled with the mixed product. This is pumped to the CLO day tank where it is transferred to the lubricator. The mixed oil is analyzed before it enters the day tank.

Figure 2.3. Product Specifications. Reprinted from MMT, 2013, Adapted from “Blending-on-Board Product sheet”. Copyright 2013 by MMT. Reprinted with permission.

The SEA-mate B3000 is the unit installed at the Maersk Kimi, specifications can be found in Figure 2.3.

Figure 2.4: Consumption feed rate against engine load. Reprinted from Sulzer, 1998, adapted from “RTA96C Operation Manual”.

Basic principles of the CLO unit

In  figure 2.5 the basic principle of the RTA96C engine CLO system can be seen, where the day tank, given by number 1, contains the blended CLO nowadays. In the figure, given by number 8.09 we can see the so-called quills, which are discussed above.

In figure 2.4 the feed rate for different engine loads are given, which in 1998 where in higher condition then the figures used with the present knowledge, condition and cost optimization.

Figure 2.5: Cylinder oil lubrication unit. Reprinted from Sulzer, 1998, Adapted from RTA96C Operation Manual. Reprinted with permission.

## Cylinder Oil

As can be read in 2.1 cylinder lubricant seals the piston rings against the rubbing surface of the cylinder liner and also minimizes friction. There are three reasons for adding cylinder oil to the cylinder namely (Hans Jensen Lubricators, 2006):

- Lubricating the cylinders in order to obtain a stable hydrodynamic oil film between the cylinder liner and piston rings,

- Maintain the cleanliness of the cylinder liner and piston rings

Figure 2.6 Recommended CLO feed rate for MAN engines. Reprinted from MAN Diesel & Turbo, 2013, Adapted from “Service Letter SL2013-571”. Copyright 2013 by MAN Diesel & Turbo. Reprinted with permission.

And neutralize the acids formed during combustion to avoid cold corrosion attack on the liner surface

The first reason is mainly done by the lubricator and the recommended feed rate. It depends on the kind of lubricator and normally varies between the 0.6 and 1.35 g/kWh nowadays. In the previous installation the feed rate was dependable on the Sulphur level in the used fuel as can be seen in the example of Figure 2.6. (Roessner, 2017)

The second reason is achieved by the chemistry of the CLO, where it is mixed to create the ideal mixture of detergent to keep the piston rings and crown clean. MAN diesel & turbo (2015) calls this the Adaptable Cylinder Oil Control (ACC) factor, which has to be optimized. This also counts for Sulzer engines and is established by monitoring the wear rate of the liner. When starting with the ACC, MAN advises to start with an ACC factor that is in the upper range and slowly reduce it when engine wear response is positive. (Nadkarni, 2015)

Figure 2.7 Recommended feed rate for variable fuel sulphur content for cylinder. Reprinted from “Wärtsilä technical journal”, 2012, by S. Jumaine, M. Zehnder and A. Wiesmann. Copyright 2012 by Wärtsilä. Reprinted with permission.

Structural modifications of the diesel engines, as stated in Wärtsilä Sulzer(2009), shows that these resulted in improvement of the tribopack(“cylinder liner – piston ring – piston”) where an example of a target would be the reducement of the CLO feed rate to appr. 1.2 g/kWh, where there is no risk of excessive wear.

Another variable is the Base Number (BN) of the oil. This variable determines the neutralizing effect of the oil and is affected by the dopes added. The acids formed in the combustion chamber are mainly because of the Sulphur. To neutralize them hydroxide is added, expressed in mg KOH/ g CLO.

As Koster (1998) already stated it is known for a long time that the presence of the sulphur in combination of oxygen where the following reaction will happen:

S+ O2 ⟹ SO2       and          2SO2+ O2 ⟹ 2SO3

As the Sulphur-3-oxide will only exist if temperatures will exceed above 675 degrees Celsius and in presence of catalyst vanadium, this will highly occur. In order to prevent the oil to become more acid the hydroxide, which is a base, will be added.

In figure 2.7 can be seen that the higher the BN number, the lower the recommended feed rate is, needed to neutralize Sulphur acids. The figure shows the adjustment of the feed rate for engines running at loads lower than 60% of the MCR. Concluded from this figure can be that using the BN of 70 has the overall lowest feed rate at the different Sulphur contents. Although this is true, in paragraph 2.3 there will be discussed what problems will occur.

In order to control the engines wear condition, the strongest tool available now is the drain oil analysis. When operating with the BOB-unit the availability of neutralizing product, KOH, should be measured for the adjustment of feed rate optimization. Used oil taken from the engine the scavenge air space, whether a drain or another way of sampling can be used for the evaluation. In figure 2.8 the outcome of the scrape down test can be analyzed where the value of the BN has to be reviewed. When the feed rate is under 10 mg KOH/kg drain oil for example for this specific MAN engine, not enough BN particles are added in the engines operation. (Jensen, 2013)

Figure 2.8: Drain oil testing values. Reprinted from "MAN Diesel & Turbo Service letter", May 2013. Copyright 2013 by MAN Diesel & Turbo

Another test for the determination of the optimum feed rate for the specific engine is the feed rate sweep test, where the engine is running for some days on different feed rates for 24 hours, and a drain oil sample is taken every end of a specific feed rate.

In combination with the drain oil test, also Iron content measurement can be performed for the engines. Tests performed on the Wärtsilä RTA96C, the same engine as the in the case that will be discussed, showing a reduction of 17% Iron content at a MCR of 45%, up to 48% in the higher loads.

As the first question in this research is about the costs made by the operation with the unit, this will be referenced with the situation where the bob unit was not installed and no actual currencies will be used, but consumption will be used as parameter.

On board the vessels a matrix, such as added in appendix 1, is used. In this matrix we can calculate the feed rate for the vessel with and without BOB-unit. In this case the feed rate is a constant at higher loads, only for the lower loads more CLO is needed to fulfill with the engines requirements.

## Wear mechanisms

Literature study shows that failures were found before on cylinder conditions, caused by the combination of the type of fuel and the usage of cylinder oils influencing the lifetime of the engine components. As stated in 2.2 one of the cylinder oil goals is the neutralization of the acids formed in the combustion chamber.

Corrosive

Normal Abrasive

To first understand the damage that can be done to the cylinder components the term wear is defined as “the damage to a solid surface that generally involves progressive loss of material and is due to relative motion between that surface and a contacting substance or substances” (ASM, 1992). Figure 2.9 shows the types of wear phenomena that will be discussed in this chapter

### 2.3.1 Cold corrosion

Figure 2.9: Types of liner wear. Reprinted from A.P.Moller-Maersk, June 2009, by J. Dragsted. Copyright by A.P.Moller-Maersk, June 2009. Reprinted with permission.

Adhesive/Scuffing

Adhesive

Cold corrosion or corrosive wear, due to the slow steaming, will result in cold cylinder liner surface below the dew point where combustion gasses condense and tiny droplets of sulphuric acid. This will corrode the liner surface and eventually damage the piston ring coating. (Maersk Fluid Technology, ND)

Figure 2.10: Dew-point phase diagram  of Sulphuric Acid. Reprinted from Handbook, Steam, April 2007, by ir. W.M.M. Huijbregts.

As this temperature is normally measured on the separate cylinders control is possible. Remaining will be challenging the thermal dynamically of the sulphur acid dew point, which calls for higher liner wall temperatures. (Marine Engineer Review, 2014)

From the information known for exhaust boilers where the acid corrosion gives problems when the temperature is too low, stated can be that this will in the very first beginning of the process will bring problems to the cylinder. The dew point of the gas stream, which lays approximately at 140°C, water and acid which are generated are absorbed on the liner material and will exaggerate the corrosion effects. (Nedlloyd, 2002) Figure 2.10 shows the temperature (vertical) where the first formed condensate is formed depending upon the concentration percentage (horizontal) of the sulphuric acid in the gasses. (Land, 1977)

Corrosive wear, or SSW, due to condensate water carry over from the scavenge air space to a cylinder has already been prevented by the placement of adequate condensate drainage, the main factor for corrosive wear will be the cold corrosion (RTA service letter 74, 2006) During slow steaming lower temperatures will come closer to the dew point in the engines cylinder, causing condensate.

As stated by Marine Engineer Review (2014) the operation with excessive feed rate is not successful at combating cold corrosion as a lot of CLO is already scraped down into the scavenge air space and reacted adequately. In order to monitor how effective the neutralizing is the piston underside scrape-down oil should be monitored and the residual base number can be maintained.

For new engines higher HT cooling water temperatures are in advanced systems, but this is not possible for existing engines. (Marine Engineer Review, 2014) On the other hand, Wärtsilä came with the isolation of the cylinder liner years ago, but this was again corrected to the old situation.

### 2.3.2 Adhesive wear

Another failure mode occurring in the engines’ cylinder due to the lack of a proper oil film is adhesive wear, which will result in contact between metal surfaces.

If this is severe wear the term scuffing will be used and can be seen by the naked eye. This wear is mostly found by sailing with loads above 60 %. As this is usually caused by a failure of the water drain in the cylinder and the temperatures of the intake air and cooling water temperatures, other solutions are found to deal with these problems (Wärtsilä Sulzer, 2009). APMM (2009) states that, although the cylinder oil has an optimal TBN for the fuel used, the thermal stability is a very important factor of how much deposit will be formed in the cylinder, where deposits will consequence the ability to lubricate on that part of the liner surface.

### 2.3.3 Abrasive wear

Another type of wear is the abrasive wear, caused mostly by impurities of the fuel or charging air. Or by other particles produced by adhesive and corrosive wear. (Jørn Dragsted, 2009) This will result in micro-cutting and brittle fractures. For the abrasional wear the phenomena which can be visually seen will be scratches, grooves and ripples (Kovaříková, 2009).

# Research design

In this chapter, the research design, respectively the method of the research within this thesis will be discussed. First there will be explained how the problem will be researched, secondly the used research measurements will be explained and finally the result will be analyzed. In order to obtain reliable and valid research, also the research method will be verified.

## 3.1 Research methods

In order to answer the main and sub questions for all ships who have the BOB-unit installed the used case will be the Maersk Kimi, where in chapter 2.1 more information can be found about the vessel.

For this research a lot of data must be obtained on board the vessel by using the data that has been saved before the installation of the unit and afterwards. In general this data of fuel and lubrication oil consumption will be used in the quantitative part of this research, whereas in the qualitative part experience of the engineers with the unit on board of vessels and failures found in the past will result in the answer to the problem defined. The combination of the two methods, called “the mixed method” will result in a research outcome that will answer the main question asked. (Verhoeven, 2015)

### 3.1.1 Method for sub question 1

As spoken above, the research will be executed partly quantitative. In order to answer the first sub question “How did placement of the BOB unit lead to a cost reduction on fuel and lubrication oil?” the data acquired the last couple of years will be needed on fuel, system oil and cylinder oil consumption will be needed.

By comparing the fuel consumption before and after placement of the unit, differences can be seen in the fuel consumption, as stated by MMT. Actually fuel consumption is a variable that is dependable upon a lot of variables (KRAL, 2013), and savings caused by the placement of the unit are not easily filtered out, especially as the savings by the refreshment of system oil is said to be a maximum of 1.5%.

Actually the total use of CLO must be analyzed, when keeping in mind the engines’ running hours and load over a certain period. In order to do so a comparison has to be made with the engines manufacturers advised feed rate and the actual feed rate. Not only this can be measured, but also the change in viscosity over a period of time of the lubricating system oil will show the changes.

As the vessels schedule has changed over time, an analyses will be made comparing the differences of %-MCR that have been seen in this period. To do so the years 2010, 2014 and 2016 will be compared, as a good indication of changes that can be seen of load, respectively speed, caused by the fact of slow steaming.

### 3.1.2 Method for sub question 2

In order to answer this question the following methods will be used.

The trend that can be seen of the wear of piston rings, or the coating thickness, is a parameter that show us whether the oil film is proper. As discussed in the theoretical framework the wear of cylinder components will reduce by adding the right amount and correct BN CLO. In this part of the research the archived data will be used to see an actual trend in the wear of pistons rings before and after installation of the unit.

In relationship with the trend of the piston ring wear, also a trend can be made based on pictures takes of the cylinder liner, piston and piston rings over the last 2 years. In this case a file will be made showing the trend seen in time of the current condition of these components. This will be in relation with the running hours and average running load of the engine.

This will be done using the following method:

- The energy meter of the ME is registered every day at sea and at the end and start of sea voyages.

- Over the period of time we can calculate energy produced by the engine over a certain period of time in kW. Then the average load during the period between inspections is calculated and analysis on the wear can be made.

To inspect if cold corrosion is not happening on the cylinder just by the wrong or right adjustments in cylinder oil, also the cylinder liner temperature will be measured and analyzed as this is a very important factor in this case (see 2.3). Also an engine specific graph will be made, were we can see the temperature of the liner at a certain load.

## 3.2 Data collection

In order to measure the asked data, measuring devices are used in this research. Other data is gathered from the ordering quantities and prices of lubrication oils.

Measuring devices that are used will be:

- Flow meters

- Feeler gauge

- Cylinder pressure indicating device

- Energy measurement device of ME

Also previous years pictures are takes from the cylinder liners, which will be used to see a trend in the usage of the blended cylinder oil.

## 3.3 Reliability and Validity

As discussed in 3.1, measuring savings in fuel and lubrication oil and drawing conclusions from this information is a difficult task. As these consumptions are dependable upon various factors.

A few factors are already seen on the Maersk Kimi which could make the outcome of data processing of the consumptions not reliable (KRAL, 2013). These factors are:

- Measuring differences in the bunkered oil between the oil provider and the ships’ measurement at soundings. The provider of the oil will state that the received oil matches with the measurement system on board the bunkering vessel. Soundings done on board will actually show that these values do, most of the time, not match.

- Changes in sailing schedules show different factors such as bad weather, temperature difference, air pressure, currents etc.

- For the cylinder liner temperatures gauges are used who are reliable for the actual liner temperature as they are placed closely to the liner surface.

Measurements in lubrication oil are actually more reliable. Also differences in received oil can be seen, but this will be of less influence because of smaller amounts, especially the high BN product.

The measurements done at the piston rings are done throughout the year by trained officers, so assumed can be that correct measurement is done. When certain measurements highly deviate from them in-between, there will be explained whether the measurement is taken into account of left out as a blunder.

As all communication about the unit is documented, concerning information of the placement, usage and savings, this will be analyzed along with the questions asked on board the vessel. The outcome of this research will provide reliable and valid information about the unit and its’ goal, as professionals, such as chief and other engineers, will give answers to the questions asked.

# Results

In this chapter the results will be discussed for the research that has been done on board of container vessel Maersk Kimi.

## Cylinder oil

In this paragraph the results of the research done for the answering of the first sub question will be explained and analyzed. The chapter is separated into 3 parts, where the first is mainly about the reduction of CLO since the installation of the BOB unit, the second part with a viscosity analyses where can be found out that this will result in fuel reduction and the last part about the changes in load, stating that above is strongly depended on the engines load instead of only taking running hours as a variable.

## Reduction of CLO consumption

| Vessel | Maersk Kimi |
| --- | --- |
| Port | Jebel Ali |
| Day tank delivery date | 2nd of May 2015 |
| Commissioning Port | Bremerhaven |
| Commissioning Date | 3rd of June 2015 |

March 2015 the approval of the BOB unit came, where the K-class vessels were supplied with the unit and had to fit the pipework, tanks etc.

As the cylinder oil from previous bunkering was then still available and the project was still in testing phase, in this research the complete year 2016 and 2017 until June will be taken as data for the new consumption of cylinder oil.

Figure 4.1: Example of Fuel Book

In the “fuel books” available on board complete records are made every day where load and consumptions are being recorded, see figure 4.1 as an example. This is done by the Chief engineer mostly daily at 10.00 LT during a sea voyage. These are used to calculate the consumptions on a daily basis, and used to report to the Maersk Ships Performance System.

Figure 4.2

With the data received from the previous year calculations are made where the cylinder lubrication oil consumption in grams per kilowatt-hour is expressed against the load at that time. For the years complete calculations were made and few mistakes in readings (see chapter 5) were filtered out resulting in the following figure (4.2).

Here can be seen that during sea voyages 66 %( 42 MW) of the MCR is the maximum load during the year 2016. During this year only heavy fuel oil is used containing sulphur levels between 3.22 -2.66%.

Figure 4.3

In the sea voyages in the beginning of the year a lower feed rate is used as can be seen in figure 4.3. This happened caused by a large amount of emergency CLO. Stated can be that during operational time of the BOB-unit, MMT found out that the already mixed 70 TBN oil, used when failure of the unit occurs, will expire after one year of rest in the tank. This is caused by the particles separating and settling at the bottom of the tank and will not be further discussed in this chapter.

Figure 4.4

The values that are measured are conform the cylinder lubricating matrix, included in appendix 1, where 1.1 g/kWh consumption is the optimal consumption for higher loads (see paragraph 2.2)

After filtering out these values a clear pattern can be seen which is conform the lubrication matrix when having a bob unit installed on the vessel.

Calculations of 2014 based on the consumption of cylinder oil in the year of 2014 are shown in figure 4.4 together with the values of 2016 where the BOB-unit is used. By fitting a trend line into the data, a model is made of these 2 years.

|  | 2014 | 2016 |
| --- | --- | --- |
| Function | y = 5.2447x-0.149 | y = 28.45x-0.317 |
| R-square | R² = 0.2531 | R² = 0.7948 |

Figure 4.5 shows these models.

Figure 4.5

These models are based upon the figures in the table above, where from the R Square the spread between the different fits can be seen. Clearly deviations between the models can be seen, 2016 showing a fit which is having more stable figures.

## Changes in Viscosity

Changes in viscosity are monitored by sending samples of the main engine 3 monthly towards VISWA lab, where the samples are being analyzed and a report of the lubrication oils is sent to the ship. An example is added in Appendix 2. From these reports, starting 2008 to 2017, the graph in figure 4.6 has been made and will be analyzed. Out of the reports and fuel books recorded on board seen can be that the viscosity has changed in July ’15 to a value of 11.5 cSt. This value is searched back to be a full change of lubrication oil in the engines sump in relation with the installation of the BOB-unit. Before placement of the BOB unit higher viscosities can be viewed as the sump is less times renewed with new oil with on average a viscosity of 11.5 cSt.

Figure 4.6

Since placement of the BOB unit a “daily” make up of 90 L is calculated on average, whereas the average make up before placement of the unit only 70 L was, where in a total of 18 samples only 12 times the sump had been made up with new oil. Since placement of the unit monthly the sump is filled with new oil in order to keep the oil optimal and gain optimal performance, as described in 2.1.

Figure 4.7

When the “running hours”, which are recorded on board, of the lubrication oil are related to the viscosity, a regression can be made of the viscosity, as a relation can be seen in the change of viscosity during higher running hours of the oil. When the engine remains in a steady state a forecast is made in Figure 18, the estimated lifetime of the oil can be seen based on viscosity. This is without taking into account that the ship will go to be docked in 2018. When the oil reaches a viscosity of 160 cSt at 40 °C it is not satisfactory, as can be seen in the engines specifications and also in VISWA analyses.

Table 1 shows the values for the regression lines, where x is the running hours and y is the viscosity. A minor difference can be seen in the slope of the models, so from these 2 year sample analysis it must be stated that there is minor difference in the change to viscosity.

| VISCOSITY AS LINEAIR FUNCTION |  |  |
| --- | --- | --- |
|  | 100 | 40 |
| Before | y = 1E-05x + 10.987 | y = 0.0003x + 85.368 |
| After | y = 5E-05x + 11.598 | y = 0.0005x + 106.56 |

Table 1

## Changes in engine average load

In order to compare the data acquired in CLO consumption and viscosity in good order the differences in load throughout the years have to be researched. In Table 1 the figures of 6 years can be found, where can be concluded that the main engines load over these years have been halved, while the total amount of sea voyages has increased. In the records change of trading route can be seen in-between.

Figure 4.8 visualizes the load change in the past 6 years

|  | 2010 | 2012 | 2014 | 2016 |
| --- | --- | --- | --- | --- |
| Amount of Sea voyages | 201 | 225 | 244 | 230 |
| Average Load [MW] | 28,0 | 26,3 | 16,6 | 15,8 |

Figure 4.8

Table 2

## Cylinder performance results

Now that the results on fuel and lubrication oil reduction are being discussed, the sub question “In what way did the unit contribute in a positive influence on lifetime of engine components?” will be answered. It is divided into 3 parts.

Paragraph 4.2.1 gives the analyses of the piston rings in the previous years, where condition is mentioned, but also the piston ring clearance and later in time coating thickness is discussed.

In paragraph 4.2.2 the liner performance will be discussed, analyzed by the pictures and evaluation given.  In this case, data of only 3 of total 12 cylinders is used to give a proper random sample survey, according to the rules of statistics and are representative for the whole engine, excluded variation in maintenance plans.

In the last docking of the ship in July 2013, Tier I upgrade has been performed. In this upgrade the engine has been completely reconditioned in a way that it will comply with the Tier I requirements. In short, the compression ratio has been increased by larger compression shims, 5-ring piston crown have been changed with a new 3-ring design and fuel injection valves nozzle tips exchanged with new type. Also the liners of cylinder number 1 & 3 were exchanged with new liners, and new insulation, as will be described in 4.3. These are the major jobs carried out, other jobs carried out can be found in appendix 3 where the supervision report on board is added.

Another change that has been carried out on cylinders 1 & 3 is the exchange to Fuel Actuated Sacless Technology (FAST) nozzles. The 2 x 3 nozzles can be inserted in the prepared FAST cylinder covers. These nozzles change the time-between-overhaul from 12.000 to 18.000 running hours, as stated in Wärtsiläs’ service bulletin, give a reduction of sac-hole volume by +/-90% and giving an improved atomization.
Another cylinder liner modification was the placement of ant polishing rings in cylinder 2 &3, which scrapes of deposits off the piston top land, preventing liner polishing caused by deposits
Starting with partial reconditioned engine cylinders, in the next 5 years 2 were without BOB-unit.

## Piston ring data

In order to do any research on the piston rings of the Sulzer engine, 3 cylinders have been selected to act as samples. The chosen cylinders have to serve as a reliable and steady sample. The data covered above shows that the cylinders 4, 9 and 12 serve as these samples, as cylinders 1 to 3 deviate from the others in a way that no comparison can be made with cylinders 4 to 12.

The first measurement that is done goes back to November 2013, as can be seen in the figures on the next page at approximately 15.000 running hours. In this report only the piston ring clearance was recorded. No other information is written about engine performance.

By further analysis of the piston ring clearance data, it comes into place that the next clearance was recorded in March 2016, where the rings had about 30.000 hours of “running”.

The figures of the first measurement in November 2013 are compared with the figures of the placement of the new piston rings during docking in 2012. The clearance measured here is the same when the new piston rings were placed, where can be concluded no wear found place during the first 15000 running hours.

Figure 4.9

The graphs shown at the next page show the data acquired from the measurements taken in the past. These measurements of piston ring gap have been performed by the engineers with the help of a feeler gauge, see figure 4.9. The exact measurements can be found in appendix 4.

Clearly the wear of the gap between the top rings can be seen from the 3 cylinders, were the extreme gap clearance was more than 0.20 mm, where the accuracy of the feeler gauge has been taken into account.

From the graphs a steady increase of 0.15 mm for cylinder 4 and 9 can be found between 33.000 and 35.000 running hours, where after 35.000 hours the gap decreased by 0.10 mm. A decreased gap is not possible, so other reasons for this will be discussed in chapter 5.

Overall a stable condition of the piston land an piston rings can be seen where no extreme deviations have occurred. From this information no difference can be seen before the installation of the BOB-unit and afterwards.

## Cylinder performance

The data in the previous section is obtained by measurement with a feeler gauge. Along with this piston ring clearance measurement, pictures are made in the past years and since 2016 also reports are being made in a template and sent to the technical department of Maersk, where analysis is performed for the ship.

All information of all cylinders is stored in the server of the Maersk Kimi, but before 2015 the information was not processed into a report. This stored data of the cylinder 4 is now processed into a report which can be found in Appendix 5.

Before analysis and taking conclusions from the pictures, the quality of the stored data, made by digital camera has to be reviewed.

### Inspections

As can be expected cylinder inspections will mostly be performed under time pressure and high working temperatures. Dirt from the combustion, of course dependent on the engines’ running condition, is built up all over when the inside is checked and is normally not cleaned before the inspection is performed.

The digital camera quality is not of a high standard, nevertheless is it useful for the inspection of the cylinder. A requirement is to make a picture of the cylinder liner, where the camera will be inserted inside the cylinder when the piston is at bottom dead center (BDC). It has to be small in order to fit, as the size of a scavenge air port is limited to about 10 cm. The engines cooling water temperature in port is usually in between 65 till 80 degrees Celsius, which is no optimal situation for making a picture of the cylinder liner. As last the dirt is unavoidable to come on the camera, so in the taken pictures some blurred or vague spots create a picture which is not suitable for analysis.

For the report pictures the most optimal pictures, as in:

- Size

- Positioning

- Cleanliness

have been chosen in order to create the most reliable view on the engines condition.

This creates a situation where time, temperature and comfort are likely to influence the quality of the pictures taken. In appendix 5 the best quality pictures are selected for cylinder 4, which is representative for the cylinders 2 and 4 to 12, based on the facts stated in the beginning of this chapter.

### Analysis

In the report 8 inspection trough time are analyzed, were the first inspection reaches to 3 years back, July 2014, a year before placement of the unit and 2 year after docking. A satisfactory cylinder condition can be seen, where no signs of any wear can be spotted in the pictures. Deposits can be seen above the scavenge air ports, throughout the full report, but increasing and decreasing in time.

Piston ring pictures showing the Chrome Ceramic coating, which is until the last report fully intact. Measurement of the coating, where the measurement device came in May 2017, shows that the layer of the top ring is on average 300 µm, where new spare ring are found to have 450 µm.

Liner surface analysis shows that corrosive wear, mostly due to over lubrication is not visible in any of the inspections, before and after the placement of the unit.

Figure 4.10: Picture of cylinder 9 showing high amount of deposits and dirt.

High amounts of deposits can be found during inspections, which are not most likely not linked to the cylinder oil or BOB-unit. In figure 4.10, a picture taken in May 2017 showing the high accumulation of deposits, caused by the fuel injectors where the elements are over the recommended lifetime, as an example.

## Cylinder liner temperatures

In order to inspect whether other factors in the wear process of the main engine on board the Maersk Kimi, which are hardly differentiated from other wear mechanisms earlier discussed, the temperatures of the liners 4,9 and 12 are recorded between at all sea voyages between the period of April and June 2017.

The Maersk Kimi is installed with temperature gauges in the liner of every cylinder forward & aft connected to the ships control system. This means a total of 360 measurements have been done in order to get reliable data on the cylinder temperature at a certain engine load.

From these 20 measurements during sea voyages, with a minimum running time of 3 hours, the graph in figure 4.11, has been made showing the temperatures with a coverage from 98 to 174°C. Based from this graph and the information given in 2.3, stated can be that mostly, during this route, the temperatures of the liner will not exceed the 140°C border where condensation of the sulphuric acid will take place.

Figure 4.11

# Discussion

In this chapter the measurements, data processing and result will be discussed in order to establish a reliable and correct conclusion, which will be discussed in chapter 6.

## CLO measurement

From history files it is noted that the flow meter of the CLO is being replaced in 2010. In order to get a correct CLO consumption measurement a correction factor has been applied to the consumption. Every day the measurements are taken from the flow meter, and monthly the “real consumption” is been taken by the soundings done during bunkering.

These measurements are then compared and a correction factor is used on the CLO consumption, which is in 2017 a factor 0.96 more CLO registered by the flow meter in a relative long time, then registered by soundings. This factor in fact cannot be seen as a constant factor for all engine loads and the consumptions of sea voyages given in figure 4.8. Another variable factor might be the oil temperature that might influence the flow meters consumption register.

## Viscosity

As the start for the BOB-unit installation it was understood that the whole engine sump had to be changed with new oil. All oil of the sump was discharged to the new base oil tank and renewed. In this case there will be a situation that is not in the advantage of the engine. Accumulation of dirt will disappear quickly. This is because the high level of dispersant and detergent in the new lubrication oil. Normally the oil has to be refreshed in phases in order to slowly dilute all the dirt which is accumulated in the engine. In this case the results of the viscosity change would have been changed differently, so that a better understanding of the graph can be realized, and even a result that had been predicted when the unit was installed.

Also in the Fuel book record can be analyzed that depending on the officer engineers on board the sump has been refreshed in an irregular way. In this report the 3 monthly totals are taken in order to come to the outcome of daily make-up of the sump, but this is just the calculation and will give the reader a view of the difference before and after the placement of the BOB-unit. Some officers regularly refresh oil once a month, some don’t refresh the oil. In this case we can state other variables influence the outcome.

## Load changes

The research that is executed is gives an analysis in time. To perform a steady measurement for the factor wear it is feasible to have the same conditions over the years that will be used. As this is not the case the research will have factors that change the results in a way that it is likely to result from other causes. A main cause is the average load, which had been halved since placement of the BOB-unit.

From literature research must be taken into account that the change to lower loads leads to other wear mechanisms, such as cold corrosion which will be discussed later.

Lower load changes also automatically changes the average rotations, if no other factors have changed in the ships design. Turning lower speed of the engine means that the cylinder oil is injected less frequent, so that the neutralization of the acids in the chamber is not as effective as it should be, independent of the BN of the CLO.

## Performance

As already described in paragraph 4.2.2.1, carrying out a thorough inspection is a difficult task for the engineers on board. To start with the piston ring measurements

In the measurements a trend is seen that varies completely. This has the following causes and is completely inevitable:

- The rotation direction of the turning gear should be in a way that the ring will be pressed against either the upper or lower land of the piston. In this case the full gap is measured.

  - If any corrections with the turning gear are done in the other direction, for example when it is turned too far and the gap cannot be measured, the measurement is not reliable.

- The deposits that exist during all measurements will also be between the rings and piston land, and cannot be cleaned. In this way the gap measured will be smaller than the true gap.

- The piston ring is not fully straight, so if the measurer will only place the feeler gauge in half, he will not measure the true clearance.

- There is a personal error in all measurements

- Blunders

When taking all these factors into account when analyzing the gap clearance and the raise showed in page 21, a minor increase of the clearance can be seen in the top ring, but no link can established by the placement of the BOB-unit.

As stated in 4.2.2.1, the analysis done by taking pictures is a way of condition based monitoring where no minor faults or wear increase can be seen, only when there is a major wear increase, which is very depended on the quality of the pictures taken.

This does not say that the inspections are not profitable, as many failure mechanism can be seen from it, which have been fully described in appendix V. A major change that can be seen after placement of the BOB unit is the amount of calcium deposit seen on the piston crown, which has been decreased due to optimum cylinder lubrication.

## Cylinder liner temperature

These measurements are done in a period where the new schedule was started, and where the ship will be in the coming period. As the liner temperatures are measured closely to the cylinder liner it can be said that these measurements are reliable and show a result which is far underneath the dew-point line.

## Breakdown

June 2017, after the measurements and data processing, the BOB-unit had an incoming alarm and since then the unit is not functioning anymore in automatic mode. It is now required to blend CLO manually. Breakdown could not be solved because of a lack of spare parts, read none, on board of the Maersk Kimi. Also other vessels are not supplied with additional spare parts.

# Conclusion and recommendations

In this chapter the conclusion from the result will be given in order to answer the main question as major target. Along with this answer and the discussion in the previous section a recommendation must be done in order to decide whether “Acid in the chamber” is permanent history by BOB and savings are made by the unit.

## Conclusion

In any form there cannot be made any cost-calculations which will show the savings that are made with the unit installed, as the consumption is not a linear function. Actually what is found that after placement of the unit is that in the lower loads the consumption will be on average higher, but the outcome is a steady line were less spreading can be found. In the higher loads a lower specific consumption can be found, which is expected.

Measurements found before placement of the unit show that there is a wide spread in the fuel consumption caused by the different sorts of CLO that are used, depending upon the sulphur content of the fuel.

When the combination of savings and the viscosity and refreshment of the main engine sump are analyzed it can be stated that although no conclusions can be made based on the specific consumption, also no differences for now can be seen in the viscosity and the future trend of the viscosity. For now because 2 years is too short for taking conclusions if there will be a major change. Taking into account that there has been a major load average load change between the years 2010 to 2016, this factor is high in all conclusion and analyses that are made in this report.

What is seen on this vessel is that always one of two main engine lubrication oil separators are running, which is not necessary looking at the reports of the lubrication oil analyses. When discussing cost savings there is much lubrication oil saved when the separator is only running when necessary and with a fixed interval.

Whilst it can be said that performance inspections of the main engine cylinder components is a difficult task executed in dire straits, the cylinder condition has an overall improvement.

The improvement which is measured monthly of the piston rings shows a steady state, where no progressive wear is shown. From the visual cylinder condition analyses it can even be stated that the placement of the unit has caused less calcium deposits, seen on the piston crown. This will decrease the abrasive wear caused by particles that will be present when the feed rate is higher, which occurs when no BOB-unit is placed. Also there are no signs of corrosive or adhesive wear, caused both by an insufficient oil film. Signs of adhesive wear were spotted before placement of the unit, which now cannot be seen anymore.

Cylinder liner temperatures showing that great caution must be taken in order to prevent cold corrosion, so monthly inspection stays necessary.

## Recommendations

By given an answer to the main question, the recommendation will follow in this paragraph:

#### “Has a Seamates’ Blending-On-Board unit (BOB) on board Maersk container vessels lead to a positive result by the supposed savings and maintenance optimization, and if not can optimization be achieved yet?”

#### Positive result

At first the placement of the BOB-unit has been an investment that shows its’ positive results. From the outcome of this report it can be concluded that placement of the unit is strongly recommended for any vessel within Maersk Line, even any vessels sailing with a large 2-stroke long-stroke diesel engine.

Anyhow recommendations can be made from the in the following way:

#### Monitoring

Improvement at this vessel will be the monitoring quality, which already improved by the purchase of a coating thickness measurer, as an extra monitoring device for the piston rings condition. Other improvements must be made on the quality of scavenge air inspection, respectively the quality of the pictures taken in order to maintain a high standard inspection.

#### Reportage

Furthermore the reportage of oil figures came to the attention. It is mostly based on the consumption of fixed BN sailing, with small upgrades. Advised is a research for the improvement of this reportage, so lubrication oil figures can better be monitored.

#### ME LO separator

Another target that could be improved is the usage of the main engine sump separator. Currently it is running when the engine is running, while oil figures show that the oil is constantly in proper condition. During starting, running and stopping the separator lubrication oil is separated and transferred to the sump in any way, while this is not necessary. Advice in order to save fuel oil, lubrication oil and lifetime of the separator to only run it when considered necessary and with fixed running intervals which will be suitable for the lubrication oil condition.

#### Cold corrosion

As previous attempts for the prevention of cold corrosion by isolation of the cylinder liner failed it is recommended to research other possibilities on decreasing the chance of corrosive wear, which for older ships needs to be feasible with the remaining lifetime of the vessel.

###### Spare parts

Since the unit broke down at the time this thesis was finished, the BOB-unit was not working, caused by lack of spare parts on board. Advice is to consider a spare part inventory for the BOB-unit, as it needs OEM parts.

# Literature

ASM. (1992). ASM Handbook: Friction, Lubrication and Wear Technology.

Beker, M. (2013). Supervision Report IMO TIER I. Dubai.

Dewan, M. (2015). Cylinder lubrication system. Retrieved January 2017, from http://marinestudy.net/cylinder-lubrication-system/

Griffiths, D. (2006). Marine Low Speed Diesel engines. London: IMarEST. Retrieved January 2017

Hans Jensen Lubricators. (2006). Improving the lubrication of two-stroke engines. Marine Engineers Review(IMAREST), 1.

Huijbregts, I. W. (2007). Handboek stoom. Nassau.

Jensen, M. (2013). MAN Diesel & Turbo Service Letter SL2013-571. Copenhagen: IMAREST publication.

Jørn Dragsted. (2009). Cylinder condition. Maersk Maritime Technology.

Kovaříková, I. (2009). Study and Characteristic of abrasive wear mechanisms. TRNAVA, Slovak Republic: Institute of Production Technologies, Slovak University of Technology.

KRAL. (2013, september). Fuel Consumption Measurement for Diesel Engines. Retrieved from KRAL.at: http://www.kral.at/uploads/tx_fastce/Flow_Measurement_KRAL_Fuel_Consumption_Measurement_for_Diesel_Engines.pdf

Kuiken, K. (2012). Diesel Engines I. Onnen: Target Global Energy Training.

Land, E. (1977, June). The theory of acid deposition and its application to the dew point. Insitute of Fuel.

Leeuw, P. d. (1980). Analyse van Bedrijfskundige Systemen. Lemniscaat: Amsterdam.

Maersk Fluid Technology. (ND). Case study: Cold corrosion prevention. Retrieved January 2017, from Maersk Fluid Technology: www.maerskfluid.com

Marine Engineer Review. (2014). Corrosion concerns. IMAREST, 5. Retrieved January 2017

Nadkarni, N. (2015). Match Point. Fuel Line. Retrieved April 2017

Nedlloyd, P. (2002, July). Technische informatie. Ketels & Verdampers(Section VI ).

Roessner, M. (2017, January). High Basenumber Cylinder Oil. (V. Seijger, Interviewer)

Verhoeven, N. (2015). Doing research. Boom Lemma.

Wärtsilä Sulzer. (2009). The Tribopack for Sulzer RTA Engines.

# List of figures

Figure 1.1: Blending on Board mixing unit.	1

Figure 2.1: Cylinder schematic cross-section	3

Figure 2.2: Visualization of cylinder oil system	3

Figure 2.3: Product Specifications.	4

Figure 2.4: Consumption feed rate against engine load.	4

Figure 2.5: Cylinder oil lubrication unit.	4

Figure 2.6. Recommended CLO feed rate for MAN engines	5

Figure 2.7: Recommended feed rate for variable fuel sulphur content for cylinder	5

Figure 2.8: Drain oil testing values.	6

Figure 2.9: Types of liner wear	7

Figure 2.10: Dew-point phase diagram  of Sulphuric Acid.	7

Figure 4.1: Example of Fuel Book	12

Figure 4.2	13

Figure 4.3	13

Figure 4.4	14

Figure 4.5	15

Figure 4.6	16

Figure 4.7	16

Figure 4.8	17

Figure 4.9	19

Figure 4.10: Picture of cylinder 9 showing high amount of deposits and dirt.	22

Figure 4.11	23

# Appendices

- Cylinder lubrication matrix example

- Lubrication oil analyses report

- Supervision report of docking 2013

- Piston Ring measurements

- Cylinder condition report (Cylinder 4)

1

Figure A1.1

Appendix 1: Cylinder lubrication matrix example

Figure A1.2

Figure A1.4

Figure A1.3

# Appendix 2: Lubrication analyses report

2

# Appendix 3: Tier I upgrade

Figure A3I.1

3

Figure A3.2

|  |  |  |  |  | Cylinder 4 |  |  | Cylinder 9 |  |  | Cylinder 12 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | RING Clearance |  |  | RING Clearance |  |  | RING Clearance |  |  |
| Date | Running hours | ring hours | sulphur |  | 1(TOP) | 2 | 3 | 1 | 2 | 3 | 1 | 2 | 3 |
| 04/11/2013 | 95785 | 14488 |  |  | 0.4 | 0.4 | 0.3 | 0.4 | 0.4 | 0.3 | 0.4 | 0.4 | 0.3 |
| 11/03/2016 | 110571 | 29274 | 2.66 |  | 0.45 | 0.4 | 0.35 | 0.45 | 0.4 | 0.35 | 0.45 | 0.4 | 0.35 |
| 24/03/2016 | 110778 | 29481 | 3.26 |  | 0.3 | 0.4 | 0.3 | 0.45 | 0.4 | 0.3 | 0.4 | 0.4 | 0.3 |
| 27/04/2016 | 111369 | 30072 | 3.26 |  | 0.4 | 0.4 | 0.3 | 0.4 | 0.35 | 0.35 | 0.35 | 0.4 | 0.35 |
| 18/05/2016 | 111825 | 30528 | 3.26 |  | 0.4 | 0.35 | 0.3 | 0.35 | 0.3 | 0.3 | 0.35 | 0.4 | 0.4 |
| 22/06/2016 | 112399 | 31102 | 2.12 |  | 0.4 | 0.35 | 0.3 | 0.35 | 0.3 | 0.3 | 0.35 | 0.4 | 0.4 |
| 13/09/2016 | 113683 | 32386 | 3.32 |  | 0.3 | 0.3 | 0.25 | 0.45 | 0.45 | 0.25 | 0.45 | 0.4 | 0.25 |
| 26/10/2016 | 114409 | 33112 | 3.3 |  | 0.55 | 0.4 | 0.25 | 0.55 | 0.4 | 0.3 | 0.5 | 0.4 | 0.3 |
| 17/11/2016 | 114741 | 33444 | 3.27 |  | 0.55 | 0.4 | 0.25 | 0.55 | 0.4 | 0.3 | 0.5 | 0.4 | 0.3 |
| 26/12/2016 | 115467 | 34170 | 3.45 |  | 0.55 | 0.4 | 0.25 | 0.55 | 0.4 | 0.3 | 0.5 | 0.4 | 0.3 |
| 26/01/2017 | 116003 | 34706 | 3.45 |  | 0.55 | 0.4 | 0.25 | 0.55 | 0.4 | 0.3 | 0.5 | 0.4 | 0.3 |
| 23/02/2017 | 116356 | 35059 | 3.34 |  | 0.63 | 0.45 | 0.3 | 0.45 | 0.43 | 0.33 | 0.55 | 0.4 | 0.3 |
| 28/04/2017 | 117258 | 35961 | 2.34 |  | 0.5 | 0.35 | 0.3 | 0.45 | 0.35 | 0.25 | 0.45 | 0.4 | 0.3 |

4

# Appendix 4: Piston ring  measurements

# Appendix 5: Cylinder condition report

| Date: | 24/07/2014 | Running hours: | 100654 |  |
| --- | --- | --- | --- | --- |
| Piston rings |  | Lower liner surface |  | Liner top |
| Analysis: |  |  |  |  |
| Appear to be in good working condition, mirroring surface with no sharp edges. Ceramic coating fully intact. |  | No honed surface anymore, but with no scores of wear in any form. Only deposits, but causing no wear to the liner. |  | No signs of scuffing or other liner wear. Lubrication grooves are clearly visible and in good form along with “wet” liner surface |

5

| Date: | 03/01/2015 | Running hours: | 103074 |  |
| --- | --- | --- | --- | --- |
| Piston rings |  | Lower liner surface |  | Liner top |
|  |  | No picture taken |  |  |
| Analysis: |  |  |  |  |
| Cleaned rings appear all to be in good working condition, mirroring surface with no sharp edges. Ceramic coating fully intact. |  | - |  | No signs of scuffing or other liner wear. Lubrication grooves are clearly visible and in good form along with “wet” liner surface |

| Date: | 30/08/2015 | Running hours: | 107844 |  |
| --- | --- | --- | --- | --- |
| Piston rings |  | Lower liner surface |  | Liner top |
| Analysis: |  |  |  |  |
| Appear to be in good working condition, mirroring surface with no sharp edges. Chrome Ceramic coating fully intact. |  | Smooth surface with no scores of wear in any form. Small sign of black deposits on the left, which could be result from scuffing. Note: picture is showing 50% of the piston crown, where we can see no burnings and only slight calcium deposits. |  | No signs of scuffing or other liner wear. Lubrication grooves are clearly visible and in good form along with “wet” liner surface. Surface contains deposits which cannot be recognized. |

| Date: | 24/03/2016 | Running hours: | 107844 |  |
| --- | --- | --- | --- | --- |
| Piston rings |  | Lower liner surface |  | Liner top |
| Analysis: |  |  |  |  |
| All rings appear to be in good working condition, mirroring surface with no sharp edges. Chrome Ceramic coating fully intact. |  | Picture contains small part of lower liner. Small sign of black deposits on the scavenging ports, which could be result from scuffing. Note: showing clean piston crown |  | Picture of bad quality Showing the same deposits on the wet liner surface. |

| Date: | 13/09/2016 | Running hours: | 113683 |  |
| --- | --- | --- | --- | --- |
| Piston rings |  | Lower liner surface |  | Liner top |
| Analysis: |  |  |  |  |
| All rings appear to be in good working condition, mirroring surface with no sharp edges. Excessive deposits can be found on the piston land. Chrome Ceramic coating fully intact. |  | Signs of black deposits on the liner, probably resulting due to long period of running at low load in combination with cold corrosion. |  | Cylinder lubrication grooves visible. Signs of cold corrosion and black deposits. Not clearly if any signs for scuffing. |

| Date: | 27/03/2017 | Running hours: | 113683 |  |
| --- | --- | --- | --- | --- |
| Piston rings |  | Lower liner surface |  | Liner top |
| Analysis: |  |  |  |  |
| All rings appear to be in good working condition, mirroring surface with no sharp edges. Excessive deposits from previous analysis are gone. Chrome Ceramic coating fully intact. |  | Signs of black deposits on the liner, probably resulting due to long period of running at low load in combination with cold corrosion. Minor signs of slight abrasive/adhesive wear. |  | Upper liner surface showing deposits from scavenge ports. Slight signs of abrasive wear, as stated in the lower liner surface. |

| Date: | 28/04/2017 | Running hours: | 117258 |  |
| --- | --- | --- | --- | --- |
| Piston rings |  | Lower liner surface |  | Liner top |
| Analysis: |  |  |  |  |
| All rings appear to be in good working condition, mirroring surface with no sharp edges. Some deposits can be found on the top land of the piston. Chrome Ceramic coating fully intact. Rings free to move |  | Less signs of deposits. Picture is not clear enough for wear conclusions. |  | Upper liner surface showing deposits on the wet surface. Slight signs of abrasive wear, as stated in the lower liner surface. |

| Date: | 28/04/2017 | Running hours: | 117258 |  |
| --- | --- | --- | --- | --- |
| Piston rings |  | Lower liner surface |  | Liner top |
| Analysis: |  |  |  |  |
| All rings appear to be in good working condition, mirroring surface with no sharp edges. Chrome Ceramic coating fully intact. Rings are free to move |  | Minor signs of adhesive or abrasive wear registered. |  | Upper liner surface showing deposits on the wet surface. Slight signs of adhesive wear |